Activity and Use Limitations (AULs) fall into several categories including restrictions on land use, soil disturbance, groundwater use, and construction.  Depending on site-specific conditions, restrictions other than those listed here may be appropriate, and may be agreed upon by the Owner and the Department.

Land Use Restrictions:

- This “No Residential Land Use” restriction is often used in Covenants, and may appear by itself or in combination with other restrictions for soil and groundwater.  It prohibits all residential uses, including but not limited to schools and retirement or senior/child-care centers.  While schools and retirement or senior/child-care centers may not be considered residential uses for purposes of many local zoning schemes, these uses might not be safe under the typical risk assessment approaches used for non-residential exposures, and are therefore often prohibited as a safety measure.  Special cases can be considered on a site-specific basis.

No Residential Land Use:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, the Property currently meets the Department’s standards for non-residential use.  Therefore, contaminants of concern remaining at the Property do not pose a significant current or future risk to human health or the environment so long as the restrictions below remain in place.

The Property shall not be used for residential purposes, which for purposes of this Covenant include but are not limited to:  single family homes, duplexes, multi-plexes, apartments, condominiums, schools, retirement or senior/child-care centers, or any land use where persons can be expected to reside.

- This “Restricted Residential Land Use” restriction is a special category which allows a site with contamination above residential target levels to be used for apartments, condominiums, or similar residential uses when it is safe to do so.  This restriction would typically be used in connection with other additional restrictions.  Examples might include an apartment complex with contaminated soil capped under a parking lot, or non-volatile contaminants under a building foundation. Such uses may be currently existing or planned redevelopment.  The approach is most often used with Brownfield redevelopment.  In any acceptable scenario, actual exposures to the residents must not exceed a Residential level in order to use this provision. For example, exposed surface soil would have to meet Residential target levels so that residents could safely use green space.  The property owner, rather than each tenant, is responsible for carrying out any special management of the residual contamination to prevent exposures.  If an engineered barrier is involved, a restriction pertaining to such engineered control would also be used.  Single family homes, duplexes and similar developments (whether rental or owner occupied) are not suitable for this provision.

Restricted Residential Land Use:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, the Property currently meets the Department’s standards for non-residential use and for certain limited residential uses.  Therefore, contaminants of concern remaining at the Property do not pose a significant current or future risk to human health or the environment so long as the restrictions below remain in place.

The Property may be used for non-residential purposes, and/or multi-family residential purposes such as apartments or condominiums; provided, however, that the Property shall not be used for single family homes, duplexes, or [include as appropriate:  schools, child-care facilities, etc.].

Soil Restrictions:

- This “No Disturbance of Soil” restriction prohibits all disturbance of soil in a specified area and depth on the property, unless done in accordance with a Department-approved plan.  The “Construction Worker Notice” restriction is also used when soil at the site (or in a specific area) exceeds target levels appropriate for construction worker exposure. It is not used at all sites with soil disturbance restrictions, since safe levels of contaminants for construction worker exposure differ from those for residents and non-residents.  Note that while the Department may review and approve Soil Management Plans in connection with a “No Disturbance of Soil” restriction, the Department does not review and approve Worker Health and Safety Plans.  While the Covenant can prohibit construction work at the site (or in a specific area) unless soil is managed appropriately and appropriate measures are taken to protect workers, the Department will not approve, and the Covenant need not specifically describe, what those worker safety protective measures should be.  It is the responsibility of those proposing to disturb contaminated soils to ensure that appropriate worker safety protective measures are taken in accordance with applicable health and safety laws and regulations.

No Disturbance of Soil:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, contaminants of concern remaining at the Property exceed the Department’s standards for [non-residential use and/or construction worker exposure], but do not pose a significant current or future risk to human health or the environment with respect to non-residential uses of the property so long as the soil is not disturbed such that exposure would result.

Therefore, soil on the Property [this may be limited to certain areas shown in a map attached hereto as Exhibit ___, which is hereby incorporated by reference] shall not be excavated or otherwise disturbed in any manner

[choose one of the following:]

- without the prior written approval of the Department.  If an Owner/Transferee desires to disturb soil at the Property [in one or more of the designated areas], then such Owner/Transferee shall request permission to do so from the Department at least 30 days before the soil disturbance activities are scheduled to begin.  Based on the potential hazards associated with the soil disturbance activities, the Department may deny the request to disturb the soils or may require specific protective or remedial actions before allowing such soil disturbance activities to occur.

[or:]

- except in accordance with the Department-approved Soil Management Plan attached hereto as Exhibit ____, which is hereby incorporated by reference.

Contaminated soil may be disturbed if necessary during an emergency (such as water or gas main break, fire, explosion or natural disaster), in which case the Owner/Transferee shall ensure that notification is provided to the Department and Holder verbally or in writing as soon as practicable, but no later than 48 hours after the disturbance begins.  Any contaminated soil disturbed as part of an emergency response action must be returned to its original location and depth, or properly characterized, managed and disposed of, in accordance with all applicable local, state, and federal requirements.  Within 30 days after such emergency has been abated, the Owner/Transferee shall provide a written report to the Department describing such emergency and any response actions.

[Use when Construction Worker levels are exceeded:]

Construction Worker Notice

In the event that construction or excavation work is to be performed that may expose workers to contaminated soil on the Property [this may be limited to areas and depths shown on the map attached hereto as Exhibit ___, which is hereby incorporated by reference], Owner/Transferee shall ensure that actual notice is provided in advance, both verbally and in writing, to any person or entity performing any work that will or is likely to result in exposure to such soil, so that appropriate protective measures are taken to protect such workers’ health and safety in accordance with applicable health and safety laws and regulations.  Such notice shall include, but not be limited to, providing a copy of this Covenant [and Soil Management Plan, if applicable] to any individuals conducting or otherwise responsible for the work.  Owner/Transferee shall maintain copies of any such written notice for a period of at least three years, and shall provide copies of such records to the Department and/or Holder upon request.

- This “Engineered Controls for Soil” restriction is used to ensure that physical barriers or other systems designed to prevent exposure to contaminated soil are properly inspected and maintained, and are not disturbed, removed or inactivated.  The most common examples are a placing a cap over any soil contaminated above levels that are protective of potential uses, or installing vapor barriers or passive or active venting systems under buildings to prevent infiltration of volatile contaminants.  A Soil Management Plan can be included as an Exhibit to guide future site activities, such as inspection, maintenance and repair of the control, utility trenching and repair, installation of fence or sign posts, modification or construction of buildings, or any other anticipated activities as appropriate.  Such plan may provide for any record-keeping, Department notification, and/or Department approval of various activities.  Self-inspection items and reporting schedule requirements may also be included in the plan.

Engineered Controls for Soil:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, contaminants of concern remaining in the soil at the Property do not pose a significant current or future risk to human health or the environment so long as the engineered control(s) described below is/are maintained so as to prevent exposure, release, or migration of contaminants from the soil.

Therefore, the following engineered controls must remain in place and remain effective in accordance with the Department-approved ____[insert name of plan]____, unless and until the Department provides written approval for any modifications:

[Describe engineered control(s), and insert requirements for maintenance of engineered controls. If appropriate, refer to Soil Management Plan included as an Exhibit.  Alternatively, describe how the control (e.g. Soil Cap, Parking Lot, pavement, building slab, etc.) and contaminated soil in the area of the Property shown in Exhibit __ must be effectively maintained (as an exposure barrier, a rainwater infiltration barrier, etc.) as specified in _____[Management Plan]_____.  Or that the active or passive venting system or vapor barrier installed in a particular area of the Property must be effectively maintained according to____ [Management Plan]___.]

Groundwater Restrictions:

- This “No Drilling or Use of Groundwater” restriction prohibits the use of contaminated groundwater beneath the site.  It also prohibits drilling for other purposes that could spread the contamination to deeper aquifers.  Certain activities would be allowed under plans approved by the Department, such as the installation of monitoring wells, withdrawal of samples for analysis, or drilling of wells or withdrawal of water for remediation purposes.

No Drilling or Use of Groundwater:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, contaminants of concern remain in groundwater in one or more zones beneath the Property at levels exceeding the Department’s standards for one or more specific groundwater uses.  Such exceedances include:  [describe contaminants and what standard they exceed; e.g. benzene exceeds domestic use standard].

Therefore, in addition to any applicable state or local well use restrictions [provide citation if existing restrictions are known to apply], the following restrictions shall apply to the Property:

- Groundwater from [this may be limited to only certain contaminated zones] the Property shall not be consumed or otherwise used for any purpose, except for the collection of groundwater samples for environmental analysis purposes, collection or treatment of groundwater for remedial purposes, or collection or treatment of groundwater as part of excavation or construction activities.

- There shall be no drilling or other artificial penetration of any groundwater-bearing unit(s) containing contaminants, unless performed in accordance with a Department-approved work plan;

- Installation of any new groundwater wells on the Property is prohibited, except for wells used for investigative, monitoring and/or remediation purposes installed in accordance with a Department-approved work plan.

- This “Engineered Controls for Groundwater” restriction is used to ensure that physical barriers or other systems designed to prevent exposure to contaminated groundwater are properly inspected and maintained, and are not disturbed, removed or inactivated.  Common examples include:  (1) passive or active underground systems designed to control the movement of groundwater such as a pumping system or bentonite slurry wall, (2) systems designed to passively or actively treat groundwater such as flow-through chemical or bio-filters, or (3) vapor barriers and passive or active venting systems under buildings to prevent infiltration of volatile contaminants emanating from a groundwater plume.  A management plan can be included as an Exhibit to guide future site activities, such as inspection, maintenance, and repair of the control, utility trenching and repair, installation of fencing or sign posts, modification or construction of buildings, or any other anticipated activities as appropriate.  Such plan may provide for any record-keeping, Department notification, and/or Department approval of various activities.  Self-inspection items and reporting schedule requirements may also be included in the plan.

Engineered Controls for Groundwater:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, contaminants of concern remaining in the groundwater beneath the Property do not pose a significant current or future risk to human health or the environment so long as the engineered control(s) described below is/are maintained so as to prevent exposure to, or release or migration of, contaminated groundwater.

Therefore, the following engineered controls must remain in place and remain effective in accordance with the Department-approved ____[insert name of plan]____, unless and until the Department provides written approval for any modifications:

[Describe engineered control(s), and insert requirements for maintenance of engineered controls. If appropriate, refer to Management Plan included as an Exhibit.  Alternatively, describe how the control and contaminated groundwater in the area of the Property shown in Exhibit __ must be effectively maintained (as an exposure barrier, a rainwater infiltration barrier, etc.) as specified in _____[Management Plan]_____.  Or that the active or passive venting system or vapor barrier installed in a particular area of the Property must be effectively maintained according to____ [Management Plan]___.]

- These “No Construction Worker Exposure to Groundwater” and “Construction Worker Notice” restrictions are used when groundwater at the site (typically shallow groundwater that would be encountered in an excavation) exceeds target levels appropriate for construction worker exposure. They are not used at all sites with soil or groundwater restrictions, since safe levels of contaminants for construction worker exposure differ from those for residents and non-residents.  Note that while the Department may review and approve Soil Management Plans in connection with this restriction, the Department does not review and approve Worker Health and Safety Plans.  While the Covenant can prohibit construction work at the site (or in a specific area) unless soil is managed appropriately and appropriate measures are taken to protect workers, the Department will not approve, and the Covenant need not specifically describe, what those worker safety protective measures should be.  It is the responsibility of those proposing to disturb contaminated soils to ensure that appropriate worker safety protective measures are taken in accordance with applicable health and safety laws and regulations.

No Construction Worker Exposure to Groundwater:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, contaminants of concern remaining at the Property exceed the Department’s standards for construction worker exposure to groundwater, but do not pose a significant current or future risk to human health or the environment so long as direct exposure is prevented.

Therefore, workers conducting construction or excavation activities shall not be exposed to groundwater on the Property [this may be limited to areas and depths shown on the map attached hereto as Exhibit ___, which is hereby incorporated by reference]

[choose one of the following:]

- in any manner without the prior written approval of the Department. If an Owner/Transferee desires to conduct activities that will or could expose workers to contaminated groundwater at the Property [in one or more of the designated areas], then such Owner/Transferee shall request permission to do so from the Department at least 30 days before such activities are scheduled to begin. Based on the potential hazards associated with such activities, the Department may deny the request or may require specific protective or remedial actions before allowing such activities to occur.

[or:]

- except in accordance with the Department-approved Management Plan attached hereto as Exhibit ____, which is hereby incorporated by reference.

Contaminated groundwater may be disturbed if necessary during an emergency (such as water or gas main break, fire, explosion or natural disaster), in which case the Owner/Transferee shall ensure that notification is provided to the Department and Holder verbally or in writing as soon as practicable, but no later than 48 hours after the disturbance begins.  Any contaminated groundwater disturbed as part of an emergency response action must remain in the excavation, or be properly characterized, managed and disposed of, in accordance with all applicable local, state, and federal requirements.  Within 30 days after such emergency has been abated, the Owner/Transferee shall provide a written report to the Department describing such emergency and any response actions.

Construction Worker Notice:

In the event that construction or excavation work is to be performed that may expose workers to contaminated groundwater on the Property [this may be limited to areas and depths shown on the map attached hereto as Exhibit ___, which is hereby incorporated by reference], Owner/Transferee shall ensure that actual notice is provided in advance, both verbally and in writing, to any person or entity performing any work that may result in exposure to such groundwater, so that appropriate protective measures are taken to protect such workers’ health and safety in accordance with applicable health and safety laws and regulations.  Such notice shall include, but not be limited to, providing a copy of this Covenant [and Management Plan, if applicable] to any individuals conducting or otherwise responsible for the work.  Owner/Transferee shall maintain copies of any such written notice for a period of at least three years, and shall provide copies of such records to the Department and/or Holder upon request.

Construction Restrictions:

- This “No Construction of Buildings” restriction prohibits the construction of enclosed buildings without the Department’s written approval, and is usually used when vapor intrusion from contaminated soil or groundwater would be expected to make such a building unsafe.

No Construction of Buildings:

Based on reports on file at the Department’s offices in Jefferson City, Missouri, contaminants of concern remaining at the Property exceed the Department’s standards for [residential/non-residential use] related to [subsurface soil/groundwater to indoor air exposure for volatile contaminants] [in certain areas of the Property].

Therefore, no enclosed buildings may be constructed on the Property [in the areas shown on the map attached hereto as Exhibit ___, which is incorporated herein by reference] without written approval from the Department.  If an Owner/Transferee desires to construct an enclosed building on the Property [or in specified areas shown on the map], then such Owner/Transferee shall request, in writing, approval from the Department at least 60 days before construction is anticipated to begin.  Based on the potential hazards associated with the building to be constructed, the Department may approve the request, deny the request, or may require specific protective or remedial actions before allowing construction activities to occur.  Construction shall not be initiated prior to receipt of written approval from the Department.